Supplementary material

Reducing Heuristic Fragmentation in a Decarbonised Steel Supply Network: Repair Operator, Exact LP Completion, Epsilon-Constraint Trade-offs and a Quantum-Kernel Test

# Table S1. Sensitivity of the exact optimum

Table S1 gives the exact MILP optimum of the real-size base case (demand 1.0, no emission cap) when one input changes and all others keep their values (38 rows). 16 inputs are multiplied by 0.8 and by 1.2. 6 further cases set one input to a specific value:

- Phosphorus limit 0.040%: the limit of grade Fe 500D in IS 1786:2008, Amendment No. 1 [S17]. The main model uses grade Fe 500 (0.055%), a grade chosen by the authors. With this 0.040% limit, and with 0.8 times the main limit (0.044%), the model has no feasible solution, because the phosphorus maxima assumed for scrap are too high to meet it.

- Annual cost of heat recovery, direct rolling, scrap preheating and the gas switch, each computed with the six-tenths rule instead of linear scaling of the UNDP reference units [S8]: capital cost = C_ref × (S/S_ref)^0.6, then the same capital recovery factor at 9.08% [S6].

- DRI emission factor 1.53 tCO2/t: the imported coal and pellets case of the CEEW brief [S16]. The main model uses the domestic coal and lump ore case (2.08 tCO2/t) as a high case, because the brief reports that Ballari plants use imported coal (manuscript Section 9).

Columns: the input changed, the change, the value used, whether the model was solved, expected yearly cost, expected yearly emissions, the DRI share of the charge, and the options chosen. "Power contract" is the renewable power purchase contract tier; "solar" is the captive solar tier.

| Input | Change | Value used | Solved | Cost (billion INR/yr) | Emissions (tCO2e/yr) | DRI share | Options chosen |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phosphorus in domestic scrap (upper bound) | x 0.8 | 0.036 | yes | 5.070 | 185,719 | 0.556 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, DRI contract |
| Phosphorus in domestic scrap (upper bound) | x 1.2 | 0.054 | yes | 5.131 | 90,441 | 0.252 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, scrap shredder, imported-scrap contract, DRI contract |
| Phosphorus in imported scrap (upper bound) | x 0.8 | 0.036 | yes | 5.093 | 138,304 | 0.407 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Phosphorus in imported scrap (upper bound) | x 1.2 | 0.054 | yes | 5.112 | 104,436 | 0.298 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, scrap shredder, imported-scrap contract, DRI contract |
| Max phosphorus, Fe 500 TMT (ladle) | x 0.8 | 0.044 | no | - | - | - |  |
| Max phosphorus, Fe 500 TMT (ladle) | x 1.2 | 0.066 | yes | 5.067 | 190,391 | 0.570 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, DRI contract |
| Max phosphorus, Fe 500 TMT (ladle) | set to 0.04 | 0.04 | no | - | - | - |  |
| Max domestic scrap purchasable | x 0.8 | 48,000 | yes | 5.118 | 131,153 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Max domestic scrap purchasable | x 1.2 | 72,000 | yes | 5.081 | 136,083 | 0.400 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, DRI contract |
| Scrap handling ceiling without shredder | x 0.8 | 72,000 | yes | 5.098 | 135,737 | 0.399 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, scrap shredder, imported-scrap contract, DRI contract |
| Scrap handling ceiling without shredder | x 1.2 | 108,000 | yes | 5.095 | 120,351 | 0.349 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Road freight cost, South to South zone | x 0.8 | 3.504 | yes | 5.083 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Road freight cost, South to South zone | x 1.2 | 5.256 | yes | 5.138 | 124,942 | 0.364 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, scrap shredder, imported-scrap contract, DRI contract |
| Max imported scrap purchasable | x 0.8 | 48,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Max imported scrap purchasable | x 1.2 | 72,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Max Bellary DRI purchasable | x 0.8 | 64,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Max Bellary DRI purchasable | x 1.2 | 96,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Contract minimum offtake, imported scrap | x 0.8 | 16,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Contract minimum offtake, imported scrap | x 1.2 | 24,000 | yes | 5.099 | 128,736 | 0.376 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Contract minimum offtake, Bellary DRI | x 0.8 | 16,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Contract minimum offtake, Bellary DRI | x 1.2 | 24,000 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Take-or-pay share of contracted PPA energy | x 0.8 | 0.64 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Take-or-pay share of contracted PPA energy | x 1.2 | 0.96 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, metallic recuperator on reheating furnace | x 0.8 | 409,306.4 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, metallic recuperator on reheating furnace | x 1.2 | 613,959.6 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, metallic recuperator on reheating furnace | set to 411,859 | 411,859 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, hot charging / direct rolling | x 0.8 | 2,644,120.8 | yes | 5.095 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, hot charging / direct rolling | x 1.2 | 3,966,181.2 | yes | 5.097 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, hot charging / direct rolling | set to 2,694,336 | 2,694,336 | yes | 5.095 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, FO-fired scrap preheater | x 0.8 | 1,157,424 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, FO-fired scrap preheater | x 1.2 | 1,736,136 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, FO-fired scrap preheater | set to 842,094 | 842,094 | yes | 5.095 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, reheating furnace conversion to natural gas | x 0.8 | 1,212,760 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, reheating furnace conversion to natural gas | x 1.2 | 1,819,140 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Annualised cost, reheating furnace conversion to natural gas | set to 1,334,255 | 1,334,255 | yes | 5.096 | 131,146 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Cradle-to-gate CO2 of Bellary coal-DRI | x 0.8 | 1.664 | yes | 5.096 | 109,406 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Cradle-to-gate CO2 of Bellary coal-DRI | x 1.2 | 2.496 | yes | 5.096 | 152,885 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |
| Cradle-to-gate CO2 of Bellary coal-DRI | set to 1.53 | 1.53 | yes | 5.096 | 102,404 | 0.384 | solar 20 MW, power contract 20 MW, scrap preheating, direct rolling, imported-scrap contract, DRI contract |

# Table S2. All model inputs

Table S2 lists every input of the model (70 rows) with its value, the range used for sampling, the kind of source, the numbered sources [S1] to [S35] of the data-source list at the end of this document, and the date the source was retrieved. "Assumption" marks a value chosen by the authors, with or without guidance from a source; 24 of the 70 rows are assumptions. A dash means the value has no numbered source. The scrap emission factor follows the cut-off rule of the GHG Protocol Product Standard (WRI & WBCSD, 2011).

| Input | Value | Range used for sampling | Kind of source | Sources | Retrieved |
| --- | --- | --- | --- | --- | --- |
| Rebar output of the hypothetical plant | 120,000 t rebar/yr | fixed | assumption | - | 2026-10-14 |
| Rolling yield billet to TMT rebar | 0.969 t rebar / t billet | fixed | government regulatory | S22 | 2026-10-14 |
| Rolling mill electricity (non-melting auxiliary) | 110 kWh/t rebar | fixed | report | S8 | 2026-10-14 |
| Reheating furnace fuel energy, FO-fired pusher furnace | 1.4134 GJ/t rebar | fixed | derived | S8 | 2026-10-14 |
| Max phosphorus, Fe 500 TMT (ladle) | 0.055 % mass | fixed | assumption | S17 | 2026-10-14 |
| Max copper tolerated in reinforcing bar | 0.4 % mass | fixed | peer reviewed | S21 | 2026-10-14 |
| Metallic yield of domestic scrap in the induction furnace (scrap-based unit) | 0.9479 t liquid / t charge | fixed | derived | S8 | 2026-10-14 |
| Induction furnace electricity per t domestic scrap charged (scrap-based unit) | 611.46 kWh/t charge | fixed | derived | S8 | 2026-10-14 |
| Copper in domestic scrap | 0.25 % mass | fixed | report | S20 | 2026-10-14 |
| Phosphorus in domestic scrap (upper bound) | 0.045 % mass | fixed | assumption | S19 | 2026-10-14 |
| Upstream CO2 of domestic scrap | 0 tCO2/t | fixed | assumption | WRI & WBCSD (2011) | 2026-10-14 |
| Metallic yield of imported scrap in the induction furnace (scrap-based unit) | 0.9479 t liquid / t charge | fixed | derived | S8 | 2026-10-14 |
| Induction furnace electricity per t imported scrap charged (scrap-based unit) | 611.46 kWh/t charge | fixed | derived | S8 | 2026-10-14 |
| Copper in imported scrap | 0.25 % mass | fixed | report | S20 | 2026-10-14 |
| Phosphorus in imported scrap (upper bound) | 0.045 % mass | fixed | assumption | S19 | 2026-10-14 |
| Upstream CO2 of imported scrap | 0 tCO2/t | fixed | assumption | S23 | 2026-10-14 |
| Metallic yield of coal-DRI in the induction furnace (DRI-based unit) | 0.8503 t liquid / t charge | fixed | derived | S9 | 2026-10-14 |
| Induction furnace electricity per t DRI charged (DRI-based unit) | 654.01 kWh/t charge | fixed | derived | S9 | 2026-10-14 |
| Copper in DRI (upper bound, all tramp elements) | 0.015 % mass | fixed | standard | S18 | 2026-10-14 |
| Phosphorus in DRI (upper bound) | 0.07 % mass | fixed | standard | S18 | 2026-10-14 |
| Cradle-to-gate CO2 of Bellary coal-DRI | 2.08 tCO2/t DRI | fixed | report | S16 | 2026-10-14 |
| Imported ferrous scrap, all-India CIF unit value | 37,783.23 INR/t | triangular 32,732.38 to 37,783.23 | assumption | S11 | 2026-10-14 |
| Domestic HMS (80:20), Chennai | 32,800 INR/t | triangular 32,300 to 35,600 | assumption | S29, S30, S31, S32, S33 | 2026-10-16 |
| Sponge iron PDRI, EXW Bellary | 28,000 INR/t | triangular 26,600 to 31,300 | assumption | S13, S14, S15 | 2026-10-14 |
| Max domestic scrap purchasable | 60,000 t/yr | fixed | assumption | - | 2026-10-14 |
| Max imported scrap purchasable | 60,000 t/yr | fixed | assumption | S11 | 2026-10-14 |
| Max Bellary DRI purchasable | 80,000 t/yr | fixed | assumption | S16 | 2026-10-14 |
| Scrap handling ceiling without shredder | 90,000 t/yr | fixed | assumption | - | 2026-10-14 |
| Contract discount on imported scrap | 0.02 fraction | fixed | assumption | S30, S31, S34 | 2026-10-16 |
| Contract minimum offtake, imported scrap | 20,000 t/yr | fixed | assumption | - | 2026-10-14 |
| Contract annual fee, imported scrap | 0 INR/yr | fixed | assumption | - | 2026-10-14 |
| Contract discount on Bellary DRI | 0.01 fraction | fixed | assumption | S14 | 2026-10-14 |
| Contract minimum offtake, Bellary DRI | 20,000 t/yr | fixed | assumption | S16 | 2026-10-14 |
| Contract annual fee, Bellary DRI | 0 INR/yr | fixed | assumption | - | 2026-10-14 |
| TNPDCL HT-I industrial energy charge | 7.5 INR/kWh | fixed | government regulatory | S3 | 2026-10-14 |
| Indian grid weighted average emission factor FY2025-26 | 0.000675 tCO2/kWh | fixed | government statistics | S2 | 2026-10-14 |
| Annual output, 5 MW AC off-site captive solar | 8,322,000 kWh/yr | fixed | derived | S4 | 2026-10-14 |
| Annual fixed cost, 5 MW captive solar | 32,064,661 INR/yr | fixed | derived | S4, S5, S6 | 2026-10-14 |
| Annual output, 10 MW AC off-site captive solar | 16,644,000 kWh/yr | fixed | derived | S4 | 2026-10-14 |
| Annual fixed cost, 10 MW captive solar | 64,129,323 INR/yr | fixed | derived | S4, S5, S6 | 2026-10-14 |
| Annual output, 20 MW AC off-site captive solar | 33,288,000 kWh/yr | fixed | derived | S4 | 2026-10-14 |
| Annual fixed cost, 20 MW captive solar | 128,258,646 INR/yr | fixed | derived | S4, S5, S6 | 2026-10-14 |
| Variable cost of wheeled captive solar | 1.04 INR/kWh | fixed | government regulatory | S3 | 2026-10-14 |
| Annual contracted energy, 10 MW solar open-access PPA | 16,644,000 kWh/yr | fixed | derived | S4 | 2026-10-14 |
| Annual LTOA transmission charge, 10 MW | 21,182,666 INR/yr | fixed | derived | S5, S6 | 2026-10-14 |
| Annual contracted energy, 20 MW solar open-access PPA | 33,288,000 kWh/yr | fixed | derived | S4 | 2026-10-14 |
| Annual LTOA transmission charge, 20 MW | 42,365,331 INR/yr | fixed | derived | S5, S6 | 2026-10-14 |
| Delivered price of solar open-access power | 6.07 INR/kWh | fixed | derived | S3, S4 | 2026-10-14 |
| Take-or-pay share of contracted PPA energy | 0.8 fraction | fixed | assumption | S35 | 2026-10-17 |
| Furnace oil (grade HV), export unit value | 1,227.7 INR/GJ | triangular 772.23 to 1,784.59 | assumption | S10, S12 | 2026-10-14 |
| CO2 factor, residual fuel oil | 0.0774 tCO2/GJ | fixed | intergovernmental | S10 | 2026-10-14 |
| LNG, import unit value | 1,321.88 INR/GJ | triangular 890.96 to 1,345.54 | assumption | S10, S11 | 2026-10-14 |
| CO2 factor, natural gas | 0.0561 tCO2/GJ | fixed | intergovernmental | S10 | 2026-10-14 |
| Annualised cost, metallic recuperator on reheating furnace | 511,633 INR/yr | fixed | derived | S6, S7, S8 | 2026-10-14 |
| Reheating fuel saved by recuperator | 0.2024 GJ/t rebar | fixed | derived | S8 | 2026-10-14 |
| Annualised cost, hot charging / direct rolling | 3,305,151 INR/yr | fixed | derived | S6, S7, S8 | 2026-10-14 |
| Reheating fuel avoided by direct rolling | 1.4134 GJ/t rebar | fixed | derived | S8 | 2026-10-14 |
| Annualised cost, FO-fired scrap preheater | 1,446,780 INR/yr | fixed | derived | S6, S7, S8 | 2026-10-14 |
| Induction furnace electricity saved per t scrap preheated | 68.93 kWh/t scrap | fixed | derived | S8 | 2026-10-14 |
| Annualised cost, reheating furnace conversion to natural gas | 1,515,950 INR/yr | fixed | derived | S6, S7, S8 | 2026-10-14 |
| Annualised cost, scrap shredder with bucket/vibro feeder | 1,333,705 INR/yr | fixed | derived | S6, S7, S8 | 2026-10-14 |
| Extra scrap processed by one shredder | 10,344 t/yr | fixed | assumption | S8 | 2026-10-14 |
| Road freight cost, South to South zone | 4.38 INR/t-km | triangular 2.32 to 5.01 | assumption | S27 | 2026-10-15 |
| Road freight CO2, HCV 20-30 t GVW diesel (WTW) | 8.23e-05 tCO2/t-km | fixed | intergovernmental | S24 | 2026-10-16 |
| Road distance supplier to SIPCOT Ranipet, domestic scrap | 118.2 km | fixed | derived | S26 | 2026-10-17 |
| Road distance supplier to SIPCOT Ranipet, imported scrap | 125.7 km | fixed | derived | S26 | 2026-10-17 |
| Road distance supplier to SIPCOT Ranipet, Ballari DRI | 408.3 km | fixed | derived | S26 | 2026-10-17 |
| Furnace oil burned per net kWh saved by scrap preheating | 0.002988 GJ/kWh | fixed | derived | S8, S28 | 2026-10-15 |
| Extra rolling power with direct rolling | 10 kWh/t rebar | fixed | report | S8 | 2026-10-14 |
| RBI reference rate, July 2026 average | 95.821 INR/USD | fixed | government statistics | S1 | 2026-10-14 |

# Data sources

The manuscript and Tables 1, S1 and S2 cite these sources as [S1] to [S35].

[S1] Reserve Bank of India, Reference Rate Archive (FBIL reference rates), USD, 01-31 Jul 2026, 23 daily values. https://www.rbi.org.in/Scripts/ReferenceRateArchive.aspx. Accessed 2026-10-14.

[S2] Central Electricity Authority (2026), CO2 Baseline Database for the Indian Power Sector, User Guide Version 22.0, August 2026, Table S (FY 2025-26). https://cea.nic.in/api/cdm-report-version/view-pdf/en/82. Accessed 2026-10-14.

[S3] Tamil Nadu Electricity Regulatory Commission (2025), Suo-motu Tariff Order No. 6 of 2025, 30-06-2025, Determination of Tariff for Distribution for FY 2025-26 (eff. 01-07-2025). https://www.tnpdcl.org/static/tnpdcl/assets/files/tariff/2025/TO062025/TO62025.pdf. Accessed 2026-10-14.

[S4] Tamil Nadu Electricity Regulatory Commission (2019), Order No. 5 of 2019, 29-03-2019, Order on generic tariff for Solar power and related issues (mirror hosted by CBIP). https://www.cbip.org/Regulations2018/Data/Tamil%20Nadu/TN_Tariff_Orders/1_TN_Solar.pdf. Accessed 2026-10-14.

[S5] TNERC / TANTRANSCO notice of transmission charges to be collected from 01-07-2025 (Ref. T.O. No. 8 of 2022, Tables 6-8 and 6-9), archived by Prayas (Energy Group). https://archive.prayaspune.org/collection/5/document/1419. Accessed 2026-10-14.

[S6] Central Electricity Regulatory Commission (2026), Order in Petition No. 12/SM/2026, 23-08-2026, levellised generic tariff for FY 2026-27, Annexure I, para 11, PDF p.16 (discount factor 9.08%). https://cercind.gov.in/2026/orders/12-SM-2026.pdf. Accessed 2026-10-14.

[S7] Ministry of Corporate Affairs, Companies Act 2013, Schedule II Part C, useful lives (official India Code copy), PDF p.278 (15 years) and p.279 (20 years). https://www.indiacode.nic.in/bitstream/123456789/2114/5/A2013-18.pdf. Accessed 2026-10-14.

[S8] UNDP (2023), Low-carbon Technology Packages for Mini Steel Plants: A Compendium, New Delhi. https://www.undp.org/sites/g/files/zskgke326/files/2023-07/undp_steel_tech_compendium_-_web_version.pdf. Accessed 2026-10-14.

[S9] UNDP / Ministry of Steel / PwC (2015), Compendium of Energy Efficient Technology Measures for Electric Induction Furnace (EIF) Sector, Table 4. https://www.undp.org/sites/g/files/zskgke326/files/migration/in/EIF-tech-Compendium.pdf. Accessed 2026-10-14.

[S10] IPCC (2006), 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Vol. 2 Energy, Ch. 1, Tables 1.2 and 1.4. https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/2_Volume2/V2_1_Ch1_Introduction.pdf. Accessed 2026-10-14.

[S11] Ministry of Commerce and Industry (DGCI&S), TradeStat Monthly Import Data, commodity-wise all countries, HS 72044900 (ferrous waste and scrap, other) and HS 27111100 (LNG), quantity and Rs crore, Aug 2023 - Jul 2026. https://tradestat.commerce.gov.in/meidb/commodity_wise_all_countries_import. Accessed 2026-10-14.

[S12] Ministry of Commerce and Industry (DGCI&S), TradeStat Monthly Export Data, HS 27101959 (furnace oil, grade HV), Aug 2023 - Jul 2026. https://tradestat.commerce.gov.in/meidb/commodity_wise_all_countries_export. Accessed 2026-10-14.

[S13] Shanghai Metals Market (2026), "[SMM Steel] India domestic Steel and Scrap Prices Rise", 24 Aug 2026. https://news.metal.com/newscontent/104076202-smm-steel-india-domestic-steel-and-scrap-prices-rise. Accessed 2026-10-14.

[S14] Shanghai Metals Market (2026), "[SMM Analysis] Indian sponge iron prices rise to two-year high amid rising coal costs", 22 Sep 2026. https://news.metal.com/newscontent/104127101-smm-analysis-indian-sponge-iron-prices-rise-to-two-year-high-amid-rising-coal-costs. Accessed 2026-10-14.

[S15] Shanghai Metals Market (2026), "[SMM Steel] India Steel Prices Fall Amid Subdued Trading", 12 Oct 2026. https://news.metal.com/newscontent/104152645-smm-steel-india-steel-prices-fall-amid-subdued-trading. Accessed 2026-10-14.

[S16] Nitturu, K., Sripathy, P., Patidar, R., Elango, S., Yadav, D., Mallya, H. (2024), Decarbonising Coal-based Direct Reduced Iron Production: Surveying the Landscape in Steel Clusters of India, CEEW Issue Brief, July 2024. https://ceew.in/sites/default/files/decarbonising-coal-based-dri-production.pdf. Accessed 2026-10-14.

[S17] Bureau of Indian Standards, IS 1786:2008 High strength deformed steel bars and wires for concrete reinforcement, incl. Amendment No. 1 (public.resource.org copy). https://law.resource.org/pub/in/bis/S03/is.1786.2008.pdf. Accessed 2026-10-14.

[S18] Bureau of Indian Standards, IS 15774:2007 Sponge iron/DRI, HBI and CBI for steel making, Table 1. https://law.resource.org/pub/in/bis/S10/is.15774.2007.pdf. Accessed 2026-10-14.

[S19] Bureau of Indian Standards, IS 2062:2011 Hot rolled medium and high tensile structural steel, Table 1. https://law.resource.org/pub/in/bis/S10/is.2062.2011.pdf. Accessed 2026-10-14.

[S20] MEDALS project (EU Horizon Europe GA 101138516), Deliverable D2.1 TrampData, KTH Royal Institute of Technology, 27 Dec 2024, Table 3. https://www.medals.proj.kth.se/polopoly_fs/1.1468877.1776073100!/D2.1%20-%20Tramp%20Data.pdf. Accessed 2026-10-14.

[S21] Daehn, K.E., Cabrera Serrenho, A., Allwood, J.M. (2017), How Will Copper Contamination Constrain Future Global Steel Recycling?, Environmental Science & Technology 51(11), 6599-6606, doi:10.1021/acs.est.7b00997. https://doi.org/10.1021/acs.est.7b00997 (not readable logged out) and open Cambridge repository copy (handle 1810/266889) https://www.repository.cam.ac.uk/bitstreams/86716592-564e-414c-9337-fd92db5f55f1/download. Accessed 2026-10-14; quote re-checked on the Cambridge copy 2026-10-17 (journal p.6599).

[S22] Maharashtra Pollution Control Board (2022), Executive summary for public hearing, expansion of M.S. billets (induction furnace) and TMT bars (rolling mill), raw material table. https://www.mpcb.gov.in/sites/default/files/public_hearing/exe_summary/2022-04/ex_summary_06042022.pdf. Accessed 2026-10-14.

[S23] Ministry of Steel (2024), Green Steel Taxonomy, Gazette notification 23-12-2024, clause 4 (emission boundary). https://steel.gov.in/sites/default/files/2026-09/Notification%20green%20steel%20taxonomy%20for%20india%2023.12.2024-reg..pdf. Accessed 2026-10-14.

[S24] Smart Freight Centre / GLEC (2025), GLEC Framework v3.2, Table 13 Indian road emission intensity values, HCV 20-30 t GVW diesel average, WTW 0.0823 kgCO2e/t-km, Table 13 p.106; original PDF https://d1c2gz5q23tkk0.cloudfront.net/shrine_store/uploads/networks/3660/communication_news/13311209/61c644c3cbc7aabd60d91cff8d37cff2.pdf and landing page https://smartfreightcentre.org/news/13311209 (also republished by Climatiq). Accessed 2026-10-16.

[S25] OpenStreetMap contributors, Nominatim geocoder. https://www.openstreetmap.org/. Accessed 2026-10-14; one port location was looked up again on 2026-10-17.

[S26] OSRM (Open Source Routing Machine) public demo server on OpenStreetMap data, driving route distance to SIPCOT Industrial Complex Ranipet (OSM way 297545907). https://router.project-osrm.org/. Accessed 2026-10-17 (all routes queried again on that date; first queried 2026-10-14).

[S27] NCAER (2025), Assessment of Logistics Cost in India, prepared for DPIIT Logistics Division, September 2025, Sec. 4.6, Table 4.4. https://ncaer.org/wp-content/uploads/2025/09/NCAER_Logistic_Study_Report_Sept2025.pdf. Accessed 2026-10-15.

[S28] Bureau of Energy Efficiency, Guide Book 2 (Energy Efficiency in Thermal Utilities), Ch. 1 Fuels and Combustion, Table 1.2 (furnace oil density 0.89-0.95 g/cc). https://beeindia.gov.in/sites/default/files/2Ch1.pdf. Accessed 2026-10-15.

[S29] Tata nexarc blog, "Iron scrap rate today" (updated 9 Oct 2026), citing BigMint 10 Sep 2026 assessment of HMS (80:20) Chennai at Rs 34,300/t. https://blog.tatanexarc.com/da/iron-scrap-rate-today/. Accessed 2026-10-15. Archived copy (verified 2026-10-17): https://web.archive.org/web/20261017013645/https://blog.tatanexarc.com/da/iron-scrap-rate-today/.

[S30] BigMint (2026), "India: Scrap prices in Chennai increase by INR 600/t w-o-w amid healthy demand - 26 Feb", 26 Feb 2026; HMS (80:20) INR 32,500/t; imported HMS (80:20) $342-345/t CFR Chennai. https://www.bigmint.co/insights/detail/india-scrap-prices-in-chennai-increase-by-inr-600-t-w-o-w-amid-healthy-demand-26-feb-726825 Accessed 2026-10-16.

[S31] BigMint (2026), "India: Ferrous scrap prices rise by INR 300/t w-o-w in Chennai as steel prices improve - 5 Mar", 5 Mar 2026; HMS (80:20) INR 32,800/t; imported HMS (80:20) $355-357/t CFR Chennai. https://www.bigmint.co/insights/detail/india-ferrous-scrap-prices-rise-by-inr-300-t-w-o-w-in-chennai-as-steel-prices-improve-5-mar-728538 Accessed 2026-10-16.

[S32] Shanghai Metals Market (2026), "[SMM Steel] India Domestic Steel Market Sees Price Gains", 27 Aug 2026; scrap INR 32,300/t delivered Chennai. https://news.metal.com/newscontent/104083465-smm-steel-india-domestic-steel-market-sees-price-gains Accessed 2026-10-16.

[S33] BigMint (2026), "India: Chennai ferrous scrap prices remain firm w-o-w despite billet correction - 8 Oct", 8 Oct 2026; HMS (80:20) INR 35,600/t. https://www.bigmint.co/insights/detail/india-chennai-ferrous-scrap-prices-remain-firm-w-o-w-despite-billet-correction-8-oct-797021 Accessed 2026-10-16.

[S34] Hidayat, M. (2026), "Asian Ferrous Scrap Prices Ease Further in July 2026", Discovery Alert, 16 Jul 2026; shredded $360-365/t CFR Nhava Sheva, table credits Argus Media. https://discoveryalert.com/asian-ferrous-scrap-prices-july-2026-monsoon-demand-eaf/ Accessed 2026-10-16.

[S35] Heaven Green Energy, glossary entry "PPA" (vendor page; datePublished 2026-06-04, "Updated 8 July 2026"); minimum offtake "typically 80% to 90%" of expected generation. https://www.heavengreenenergy.com/glossary/ppa. Accessed 2026-10-17.